TECHNICAL DESCRIPTION OF FIBER OPTIC CABLE

Cable construction

| Cable type | A-DQ(ZN)2Y-(1-6)x24/A-DQ(ZN)4Y-(1-6)x24-10145 |  |  |
| --- | --- | --- | --- |
| Standards | IEC 60793, IEC 60794 |  |  |
| Cable construction - Outer sheath: UV-resistant PЕ/PA - Optical loose tube: PBT Tube - Intramodule compound: Thixotropic compound - Optical Fiber: 200 µm Single-mode Fiber according to ITU-T G.652.D/G.657.A1 - Central strength element: Glass reinforced plastic FRP-E - Water-blocking element of core: Water-swellable yarn - Rip-cord: Water-swellable yarn |  |  |  |
| Temperature ranges | Storage and transportation temperature | Installation temperature | Operating temperature |
|  | from -30 to +70 °С | from -5 to +50 °С | from -30 to +70 °С |
| Resistance to water penetration | Water resistant |  |  |
| Relative humidity at +350 C, % | 98 |  |  |
| Outer diameter, mm | 5,7±0,2 |  |  |
| Average weight of 1 km of the cable, kg | 20±5% |  |  |

Mechanical characteristics

| Calculated tensile strength, not less than, kN Dynamic | 0,8 |
| --- | --- |
| Minimum bending radius | 20D |
| Short-term crush test, kN/cm (N/10cm) | 0,05 (500) |
| Resistance to axial torsion at angle ±360° on 2 m length cable | Resistant |
| Resistance to bends at angle ± 900 with a radius equal to 20 nominal cable diameters | Resistant |
| Resistance to impact 2 J | Resistant |

Packaging and marking

| Cable factory length, km | 6 |
| --- | --- |
| Package | Drum №10 (1000х630х1000) |
| Tolerance, % | ±3,0 |
| Short lengths (customer approval) | Maximum 5% |
| Marking method | Inkjet printing |
| The accuracy of marking, % | ±0,5 |

Coloring

| Coloring of fibers in loose tube (According to Customer’s requirements color can be different). M-fiber with black ring mark. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| red | green | blue | yellow | white | slate | brown | violet | aqua | black | orange | pink |
| 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| red+m | green+m | blue+m | yellow+m | white+m | slate+m | brown+m | violet+m | aqua+m | transparent | orange+m | pink+m |

| Coloring of optical loose tube (According to Customer’s requirements color can be different). Core-filler color is natural. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| red | green | blue | yellow | white | slate |

| Cable sheath coloring |  |  |
| --- | --- | --- |
| Sheath material | PE/PA |  |
| Color | black | orange |

Product data

| Fiber count | 24 | 48 | 72 | 96 | 144 |
| --- | --- | --- | --- | --- | --- |
| Loose tubes x fibers | 1х24 | 2х24 | 3х24 | 4х24 | 6х24 |

Tensile strength value is for maximum number of fibers design