TECHNICAL DESCRIPTION OF FIBER OPTIC CABLE

Cable construction

| Cable type | A-DQ(ZN)B2Y-(1-8)x12-6329 |  |  |
| --- | --- | --- | --- |
| Standards | IEC 60793, IEC 60794 |  |  |
| Cable construction 1. Outer sheath: UV-resistant PЕ 2. Peripheral water-blocking strength element: Water swellable glass yarn 3. Optical loose tube: PBT Tube 4. Intramodule compound: Thixotropic compound 5. Optical Fiber: Single-mode Fiber according to ITU-T G. 652.D or G.652.D/G.657.A1 6. Insulation of central strength element: PЕ 7. Central strength element: Glass reinforced plastic FRP-E 8. Water-blocking element of core: Water-swellable yarn 9. Rip-cord: Synthetic yarn |  |  |  |
| Temperature ranges | Storage and transportation temperature | Installation temperature | Operating temperature |
|  | from -40 to +70 °С | from -5 to +50 °С | from -20 to +70 °С |
| Resistance to water penetration | Water resistant |  |  |
| Relative humidity at +350 C, % | 98 |  |  |
| Outer diameter, mm | 11,1±0,2 |  |  |
| Average weight of 1 km of the cable, kg | 100±5% |  |  |

Mechanical characteristics

| Calculated tensile strength, not less than, kN Dynamic | 3 |
| --- | --- |
| Minimum bending radius | 20D |
| Short-term crush test, kN/cm (N/10cm) | 0,2 (2000) |
| Resistance to axial torsion at angle ±360° on 4 m length cable | Resistant |
| Resistance to bends at angle ± 900 with a radius equal to 20 nominal cable diameters | Resistant |
| Resistance to impact 10 J | Resistant |

Packaging and marking

| Cable factory length, km | 6 |
| --- | --- |
| Package | Drum №14g (1400х1015х1400) |
| Tolerance, % | ±3,0 |
| Short lengths (customer approval) | Maximum 5% |
| Marking method | Inkjet printing |
| The accuracy of marking, % | ±0,5 |

Coloring

| Coloring of fibers in loose tube (According to Customer’s requirements color can be different). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| red | green | blue | yellow | white | slate | brown | violet | aqua | black | orange | pink |

| Coloring of optical loose tube (According to Customer’s requirements color can be different). Core-filler color is natural. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| red | green | blue | yellow | white | slate | brown | violet |

Product data

| Fiber count | 12 | 24 | 36 | 48 | 72 | 96 |
| --- | --- | --- | --- | --- | --- | --- |
| Loose tubes x fibers | 1х12 | 2х12 | 3x12 | 4х12 | 6х12 | 8x12 |

Tensile strength value is for maximum number of fibers design